TOPIC

A1

TOPIC

A1

WORKSHEET

A1-4 – CONTROL

SYSTEMS [24]

TIME TO COMPLETE: 30 MIN

TIME TO COMPLETE: 30 MIN

RECOMMENDED LISTENING:

CHEMICAL PLANT ZONE – SONIC 2

Youtube Link to Extended Version

THE COMPONENTS

Q1. For each description, state the component it describes.   [4]

| Description | Component |
| --- | --- |
| Turns a physical quantity into an electrical signal |  |
| Runs the decision that decides what the system should do |  |
| Converts energy from one form into another |  |
| Physically acts on the world — a motor, valve or heating element |  |

OPEN OR CLOSED LOOP

Q2. State whether each system is open-loop or closed-loop.   [5]

| System | Open-loop or closed-loop? |
| --- | --- |
| A microwave that runs for the time you typed in |  |
| A car’s cruise control holding 100 km/h up a hill |  |
| A street light on a timer that switches on at 18:00 |  |
| A street light with a light sensor that switches on at dusk |  |
| A drone holding its height using a barometer reading |  |

A SECOND THERMOSTAT

A greenhouse heater has a set point of 18.0 °C. It switches ON when the temperature falls below 17.0 °C and OFF when it rises above 19.0 °C.

Q3. (a) Complete the last column with ON or OFF.   [4]

| Time (min) | Temperature (°C) | Heater |
| --- | --- | --- |
| 0 | 17.4 |  |
| 10 | 16.8 |  |
| 20 | 17.9 |  |
| 30 | 18.6 |  |
| 40 | 19.3 |  |
| 50 | 18.5 |  |
| 60 | 17.2 |  |
| 70 | 16.9 |  |

(b) The first reading is 17.4 °C, which is below the set point, yet the heater is off. State why.   [2]

A GREENHOUSE

Q4. The same greenhouse also opens a roof vent when it gets too warm.

(a) Identify the sensor and the actuator in the vent system.   [2]

(b) State what the feedback signal is.   [1]

EXAM PRACTICE

Q5. Explain why an open-loop heater cannot keep a room at 20 °C when somebody opens a window.   [3]

Q6. A lift door has a safety sensor that stops the door closing on a passenger. Evaluate the use of a control loop that runs once every second for this job.   [3]