TOPIC

A1

LESSON A-1: THE CPU

AND GPU [34]

TIME TO COMPLETE: 1 HOUR

WARM UP:

WHAT YOU’LL NEED:  PyCharm  ·  a browser  ·  the CPU diagram from Lesson A1-1

Q1. From the CPU diagram we built last lesson:

(a) Which register holds the address of the next instruction to be fetched?   [1]

(b) Which register holds the instruction currently being executed?   [1]

(c) Which bus carries an address from the CPU to memory, and is it one-way or two-way?   [1]

PRIMARY MEMORY

Q2. Complete the table.   [4]

| Memory type | What it is for (one line) | Volatile? Y / N | Speed rank (1 = fastest) |
| --- | --- | --- | --- |
| Registers |  |  |  |
| Cache |  |  |  |
| RAM |  |  |  |
| ROM |  |  |  |

Q3. Cache is built in levels: L1, L2 and L3. State two ways in which L1 differs from L3.   [2]

1.

2.

CACHE HITS AND MISSES     [5 marks]

Q4. (a) Define cache hit.   [1]

(b) Define cache miss.   [1]

(c) A CPU has a cache hit rate of 90%. A cache access takes 1 ns; a miss must go out to RAM and takes 100 ns. Calculate the average memory access time. Show your work.   [2]

(d) The hit rate now drops to 50%. Calculate the new average access time.   [1]

THE FETCH, DECODE, EXECUTE CYCLE     [9 marks]

Q5. Fill in the blanks as we work through the cycle. Two blanks = 1 mark.   [5]

FETCH    1.  The address of the next instruction is copied from the __________ into the __________.

2. The MAR puts that address onto the __________ bus.

3. The control unit sends a __________ signal along the control bus.

4. The instruction travels back along the __________ bus and lands in the _________.

5. The instruction is then copied into the __________.

6. The PC is ________ so that it points at the next instruction.

DECODE  7.  The __________ interprets the opcode held in the IR.

EXECUTE 8.  The __________ performs the operation, and the result is placed in the __________.

Q6. Here’s the program you will type into LMC. Complete one row of the trace table per cycle.   [4]

In memory:   0: LDA 4     1: ADD 5     2: STA 6     3: HLT          address 4 holds 12, address 5 holds 30, address 6 holds 0

| Cycle | PC after fetch | IR | AC after execute | Memory changed? |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |

PIPELINING  [HL ONLY]   [3 marks]

Q7. (a) Name the four pipeline stages.   [1]

(b) Explain how pipelining lets a processor complete more instructions per second without running at a higher clock speed.   [2]

LAB     [8 marks]

Q8. Describe the role of the memory data register (MDR) during the fetch stage.   [2]

Q9. Explain why a cache miss slows the CPU down, with reference to the memory hierarchy.   [3]

Q10. Explain how the address, data and control buses interact during the fetch stage of the cycle.   [3]